Ceramics Dictionary

Banding wheel —	A turntable that can be revolved with one hand to turn a piece of pottery or sculpture while the other hand decorates it.

Bat — A plaster or wood disk or square slab or disk.on which a pot is thrown or is placed to dry when removed from the wheel. Also used when handbuilding.Bats are used to support clay work - either hand-built or wheel-thrown - in progress. Bats are also used to absorb water from clay in order to make soft clay firm enough to work with. (See kiln/bat wash)

Bisgue or Biscuit — The preliminary firing of unglazed ware to remove all moisture from the clay body and to a temperature sufficient to harden but not mature the body and to gain structural strength. It is usually advisable to bisque fire all ware before it is glazed and fired a second time. Unglazed ceramic ware that has been fired at a low temperature and to make handing easier during glazing.

Bisque firing	— The Preliminary firing process of firing ware at a low temperature, usually from cone 010 to 05, to produce bisque ware, prior to glazing and subsequent glaze firing.

Body or clay body — a mixture or blend of two or more clays and/or clay-like materials designed for a specific type of pottery ware.

Bone dry — the condition of unfired clay that has no absorbed moisture other than natural humidity.

Burnish — the process in which the unfired clay surface, usually when leather hard, is rubbed with a hard implement (smooth pebble, spoon, chamois), imparting a satinlike sheen to the clay surface.

Carving — decorating by cutting into the clay surface.

Casting (or slip casting)	— a reproductive process of forming clay objects by pouring a clay slip into a hollow plaster mold and allowing it to remain long enough for a layer of clay to thicken on the mold wall. After hardening, the clay object is removed.

Ceramic — any object formed from earthly materials hardened with the aid of heat.

Clay — a decomposed granite-type rock. To be classed as a clay the decomposed rock must have fine particles so that it will be plastic. Clays should be free of vegetable matter but will often contain other impurities, which will affect their color and firing temperatures. They are classified into various types, such as ball clays, fireclays, and slip clays. it is also used as a source of alumina and silica in glazes.

Coiling — a hand method of forming pottery by building up walls with ropelike strands of clay to build hollow, thin-walled ceramic forms.

Combing — a method of decoration developed by dragging a coarse comb or tip of a feather over two contrasting layers of wet clay slip or glaze.

Cones — tall, slender, three-sided pyramids made of clay and glaze constituents which bend or melt at a given temperature in a kiln. Cones are placed in a kiln where they can be observed from the outside when the kiln is hot and are used as a guide to the finishing or end of firing. The numbers begin at 022 (1085 F), for measuring the lowest temperature, and proceed to higher temperatures through cone 01, cone I and on up to cone 42 (3659 F). Popular temperature ranges are cones 06 and 04 for earthenware, cones 4 and 5 for hard earthenware for soft stoneware, and cones 8 to 11 for stoneware and porcelain.

Crawling — a defect in the glazing process in which the glaze gathers in lumps leaving portions of the body exposed. Many glazes when not fired high enough will crawl. Can also be caused by too heavy application of glaze.

Crazing — an undesirable and excessive crackle in the glaze, which penetrates through the glaze to the clay body. it should be remedied by adjusting the glaze or body composition to obtain a more uniform and contraction ratio.

Centering — The act of forcing a lump of clay by hand into a symmetrical form at the center of a spinning potter's wheel in preparation for throwing pottery.

Chuck —An open container used to hold work in place, while trimming on the wheel.

Dipping — the application of glaze or engobe to clay ware by immersion. The piece is dipped in the mixture just long enough to cover the desired area with an even coating of glaze or engobe.

Dry foot — the bottom of a ceramic object which has been cleaned of all glaze before firing.

Engobe — Originally, the term referred to slip that is applied over the entire surface of a piece of pottery or sculpture to change the color and/or texture of the clay body. The term now often refers to a prepared slip that is halfway between a glaze and a clay; contains clay, feldspar, flint, a flux, plus colorants. May be used on bisque ware. See slip

Foot –The ring like base of a ceramic piece, left unglazed, usually heavier than the surrounding body.

Fettling knife — A long, tapered knife used for trimming clay, and removing the fettle from a cast.

Firing

—	Heating pottery or sculpture in a kiln or open fire to bring the clay or glaze to maturity. The temperature needed to mature a specific clay or glaze varies.

Glaze —	Any vitreous coating that has been melted onto a clay surface by the use of heat. Made of fine-ground minerals that, when fired to a certain temperature, fuse into a glassy coating. Glazes may be matt or glossy, depending on their components

Glaze fire — A firing cycle to the temperature at which the glaze materials will melt to form a glass- like surface coating. This is usually at the point of maximum body maturity, and it is considerably higher than the bisque fire. However, delicate porcelain, subject to warping, is often fired high in the bisque with a lower glaze fire.

Grog –-Clay that has been fired then crushed to form a coarse, medium- grained or fine sand-like material. Grog can be of any color. It can be added to clay in amounts up to 30 percent. The addition of grog to clay reduces shrinkage, reduces drying or firing cracks, reduces warping, and enables the potter to use thicker areas of clay without the risk of blowing up the ware in bisque firing. Grog can add a decorative texture to the clay.

Greenware - clay objects which have been dried, but not fired.

Incising - engraving a decoration into unfired clay.

Hand building-general word for all forms of making pottery by simple forming systems such as thumb pots, coiling or pulling up from a mound of clay.

Kiln	—	A furnace or an oven built of heat-resistant materials for firing pottery or sculpture, sometimes referred to as a kiln.

kiln /Bat wash — A protective coating of refractory materials applied to the surface of the shelves and the kiln floor to prevent excess glaze from fusing the ware to the shelves. An inexpensive and effective wash can be made from equal parts of flint and kaolin.

Kick wheel — The traditional potter's wheel which is powered by kicking a lower wheel or by pushing a treadle back and forth with the feet.

Kneading	—	Working clay with the fingers or the heel of the hand in order to obtain a uniform consistency. working clay on a surface with the palms of the hands to remove air and obtain uniform consistency.

Leather hard	—	The condition of a clay body when much of the moisture has evaporated and shrinkage has just ended, but the clay is not totally dry Carving, burnishing, or joining slabs are often done at this stage.

Maturity - the temperature or time at which a clay or clay body develops the desirable characteristics of maximum nonporosity and hardness; or the point at which the glaze ingredients enter into complete fusion, developing a strong bond with the body, a stable structure, maximum resistance to abrasion, and a pleasant surface texture.

Mold - a form, generally made of bisque-fired clay or plaster of paris, which is used to reproduce any number of identical copies of an original design. Various types of molds include slip molds, press molds and hump molds or waste molds.

Oxidation (oxidizing firing)	— The firing of a kiln or open fire with complete combustion so that the firing atmosphere contains enough oxygen to allow the metals in clays and glazes to produce their oxide colors. Electric kilns always produce oxidizing firings unless reducing materials are added. Bright and clear low-fire colors are often associated with glazes and clays fired in an oxidation atmosphere.

Oxide - a compound containing oxygen and other elements. Sometimes refers to metallic chemicals used for coloring clays or glazes.

Pinch forming - the technique of shaping clay objects by compressing the clay between the thumb and fingers.

Plasticity - the quality of clay that allows it to be easily manipulated and still maintain its shape into different forms without cracking, crumbling or sagging.

Porcelain - a hard, vitreous, nonabsorbent clay body, white or gray in color, that rings when struck and translucent when thin.

Press mold — Any mold made from plaster, fired clay, wood, or a found object into which damp clay can be pressed to reproduce the shape of the mold.

Reduction (reducing firing, reduction atmosphere)

A firing in which insufficient air is supplied to the kiln for complete combustion. Under these conditions, the carbon monoxide in the kiln combines with the oxygen in the oxides of the clay body and glaze, causing the oxides to change color. Commonly associated with highfire stoneware, porcelain, and raku.

Resist - A method of applying a covering material such as wax, latex, or special luster resist to bisque or glazed ware, then coating the piece with a glaze or a second glaze. The resist material will not accept the glaze so that on firing, the color of the covered area will remain intact.

Rib - a tool of wood, bone, or metal that is held in the hand while throwing to assist in shaping the pot or to compact the clay.

Slip—A suspension of clay in water used for casting pottery or sculpture in moulds. Slip (sometimes called Engobe) can also be used for painted decoration or for the sgraffito technique.. As a noun, slip means liquid mud only to potters and ceramists.

Sprigging	— The process of attaching low-relief decorations of damp clay onto already formed greenware.

Slab construction - a handbuilding method in which forms are created by joining flat pieces of clay. The pieces are thinned and flattened with a rolling pin or slab roller.

Slurry - a term describing clay of soft, paste-like consistency.

Stain - sometimes a single coloring oxide, but ususally a combination of oxides, plus alumina, flint, and a fluxing compound. This mixture is calcined and then finely ground and washed. The purpose is to form a stable coloring agent not likely to be altered by the action of the glaze or heat. While stains are employed as glaze colorants, their chief use is as overglaze and underglaze decorations and body colorants.

Stoneware - a high-fire nontranslucent clay body which matures between cones 6 and 10, with slight or no absorbency. It is usually gray in color but may be tan or slightly reddish. Stoneware is similar in may respects to porcelain, the chief difference being increased plasticity and the color, which is the result of iron and other impurities in the clay.

Test tiles — Small tiles made of clay used to test clay bodies in the kiln or to test glazes on a specific clay body.

Throwing — Forming objects on the potter's wheel using a clay body with plastic qualities .

Updraft kiln — A kiln in which the heat goes up through the chamber and is vented through the top of the kiln.

Ware	—	A general term applied to any ceramic - earthenware, stoneware, or porcelain -in the green, bisqued, or fired state.

Wash - coloring oxide mixed with water. Sometimes used over the glaze or under the	glaze.

Warping — Changes in the form of a clay body. Warping of ware can occur during drying or firing if the walls are built unevenly or if drying or firing is uneven or ware is improperly supported during firing

Wax resist — a method of decorating pottery by brushing on a design with a hot melted wax solution or a wax emulsion. This will prevent an applied stain or glaze from adhering to the decorated portions. The wax may be applied to either the raw or bisque ware, over or between two layers of glaze. It can also be used to retard the drying of leather hard ware.

Wedging table — A table of plaster, wood, or concrete, often covered with canvas, on which clay can be wedged. A stretched wire attached to the table allows one to cut the clay to check for air bubbles, lumps, or lack of homogeneity

Wedge or wedging - a method of cutting, beating and kneading a mass of clay like bread dough to expel the air and make the whole mass homogenous. Wedging is extremely important. All clay should be wedged before beginning any construction process.