As a beginner working with clay, the following flow chart gives a handy reminder of the stages involved in the ceramic process. Once you have familiarised yourself with the process by actually doing it, you will no longer require the chart.

The link below gives a simple flow chart of the ceramic process.

http://process.arts.ac.uk/content/flow-chart-ceramic-process

Brief explanation of the ceramic process flow chart

Raw clay ( words in blue link to technique and/or process)

-can be formed by ‘throwing’ on a potters wheel, or ‘coiled’ using long ‘ropes’ of clay. Or simply ‘pinched’ into a bowl shape, moulded into sculpture, pressed into a mould, the list is infinite.

Raw clay starts drying from the minute you expose it to air, so part of the process might require adding moisture back into the clay using a spray gun/sponge with water. Work can also be kept damp from day to day by wrapping in soft polythene.

The leather hard stage is when the clay has lost some moisture and feels hard and cold to touch. At this stage the thrown pieces can be turned, the coiled pieces scraped smooth, press moulded pieces can be joined together and liquid colour called slip can be applied.  Many other building and decorative techniques also happen at the leather hard stage.

Bone dry is when clay can be bisque or first fired. This is the stage at which clay has lost all the water that can evaporate. It will feel dry and room temperature. This is also the stage at which clay can be recycled, for example if work is broken accidentally. Bone-dry clay is ready for firing.

Bisque (First) firing. Generally a bisque firing is when work is cooked slowly in a kiln, in order that the chemically bonded water in the clay molecules leaves slowly, the rate of rise of temperature is 100 degrees per hour until 600 centigrade then on to 1000 centigrade.

After firing, biscuit slips or glaze can be applied.

Biscuit slips are then bisque fired again. Bisque ware is vitrified clay, it has become ceramic and this process cannot be reversed.

Glaze (second) firing

Glaze is the shiny glass like layer that can make ceramic waterproof or give colour or decorative effects. Glaze is fired according to the type of glaze, which contains ingredients that melt at particular temperatures. Earthenware has a range of 1020-1120 degrees centigrade.

Stoneware glaze has a range between 1200- 1280 degrees centigrade.

Other glazes such as raku melt at even lower temperatures 1000 degrees centigrade.

Transfers

Transfers are commonly seen pictures or photographs on factory ware.

Transfers are applied onto glazed ware and have to be fired again.

If you have access to screen-printing facilities you can make your own, or send digital images to a specialist supplier.